Optotrak Startup Sequence

- Turn on powerrails.

- Turn on computers.

- Turn on ODAU unit (when used) and wait for it to beep. If the ODAU unit is not used, don’t turn it on.

- Turn on Sensor Unit 1 and wait for it to beep. Remove caps.

- Turn on  Sensor Unit 2 (when used) and wait for it to beep. Remove caps. If sensor junit 2 is not used, don’t turn it on.

- Turn on Control Unit and wait for it to beep.

- Start First Principles and load your First Principles experiment. The used units will beep.

- Align Sensor Units (when using more than one sensor unit).

- Define coordinate system.

- Connect IRED markers.

- Place IRED markers on subject, AFTER BEING SURE THAT THE IREDS ARE NOT GETTING TOO HOT!