SET CONTENTS:

- Insta360 X3 case (insert replaced by acoustic foam cut to size)

- Insta360 X3 camera + neoprene cover (included with camera)

- Insta360 X3 spare battery + case (included with battery)

- 2* Sandisk Extreme 256GB microSD cards (1 in camera, 1 in battery case)

- KT-30 mini stand

- Insta360 invisible selfie stick (sets 1-3: 70cm, sets 4-5: 85cm)

- Ugreen USB charger (sets 1-3: black, 15W, sets 4-5: white, 20W)

- USB-A to USB-C cable (included with camera)

- Cleaning cloth (included with camera)

- MicroSD to SD adapter (included with SD cards)

CAMERA SETUP:

1. Download, install and open Insta360 app on a phone. No account needed.

2. Power on camera (button on side)

3. Set language to English

4. In app, go to settings -> Connect device

5. Follow steps to connect, confirm on camera when prompted.

6. If prompted, update firmware.

7. When prompted, activate camera. Do not register.

8. In app settings -> Set Wi-Fi Password -> Change to: Radboud360*

9. In app settings, make sure Leica and "Shot On" watermarks are turned off.

10. In app settings -> Nadir Logo -> make sure no logo is selected.

11. On camera, make sure you are in 360 video mode, and set resolution and frame rate to 4K/25fps

12. On camera, press Q button (under power button)

13. If prompted, skip tutorial.

14. Remove default sports preset (... -> Delete)

15. Overwrite Customize1 preset with current video settings (... -> Overwrite)